FRW4_0_RTOS UI Developer API Guide

Motor, Stepper, Motion, Telemetry, and UI Integration APIs

Generated from the latest accessible main branch state of Mihavin/Open-loop, folder FRW4_0_RTOS. Intended for TouchGFX/UI integration and application-level command design.

# 1. Purpose and scope

This document summarizes the public APIs that a UI developer can use or expose through a UI service layer. It is focused on FRW4_0_RTOS and the current modules in the last commit: BLDC FOC control, TMC2130 stepper control, acceleration-limited stepper motion, telemetry, buzzer, LEDs, and TouchGFX lifecycle functions.

Do not call low-level timer, GPIO, ADC, or SPI HAL functions directly from UI screens.

Prefer command/request APIs for motor actions; these are designed to avoid direct mutation of live control state.

The UI should read state through getters or a future consolidated UI snapshot API.

Blocking APIs are marked. Avoid calling blocking functions from TouchGFX event handlers.

| Area | Primary header | UI usage |
| --- | --- | --- |
| BLDC motor control | Core/Inc/motor_foc.h | Safe commands, mode selection, arm/disarm/stop requests, state/current/speed/position getters. |
| TMC2130 low-level driver | Core/Inc/tmc2130.h | Bring-up/status/debug. UI should normally use the motion layer rather than direct STEP clock calls. |
| Stepper motion layer | Core/Inc/tmc2130_motion.h | Recommended stepper UI control: enable, move, stop, emergency stop, BLDC sync. |
| Telemetry overlay | Core/Inc/telemetry.h | Debug overlay only; production UI should use TouchGFX widgets/model values. |
| Buzzer | Core/Inc/buzzer.h | Audible feedback. Blocking beep is not recommended in UI thread. |
| LEDs | Core/Inc/leds.h | Debug/status indicators. |
| TouchGFX lifecycle | TouchGFX/App/app_touchgfx.h | Generated lifecycle/process functions; normally called by GUI task, not screens. |

# 2. UI calling rules

| Rule | Meaning |
| --- | --- |
| Use request APIs for motors | Motor actions such as arm, disarm, stop, emergency stop, set mode, and set setpoints should be requested through public APIs, not performed with direct HAL writes. |
| Do not call periodic engine functions from UI | MotorFOC() and TMC2130Motion_Task() are scheduler/owner-loop functions. They should run from dedicated tasks/timers. |
| Avoid blocking calls in UI | Avoid TMC2130_MoveBlocking() and Buzzer_Beep() from TouchGFX callbacks. |
| Use getters for display | UI should display state using getters or a UI snapshot wrapper. |
| Keep low-level driver calls out of screens | TouchGFX code should call a service/facade, not SPI/TIM/GPIO-facing functions directly. |

# 3. Runtime task map

| Task / function | Current role | UI relevance |
| --- | --- | --- |
| MotorTask / StartMotorTask | Initializes and starts BLDC FOC, then calls MotorFOC() every 1 ms using vTaskDelayUntil(). | UI must not call MotorFOC(); use MotorFOC_* request/setter APIs. |
| GUITask / StartGUITask | Runs MX_TouchGFX_Process() and Telemetry_Update() periodically. | TouchGFX screens execute in this context; avoid long blocking calls. |
| HIDTask / StartHIDTask | Reads UI encoder and updates BLDC setpoints based on requested mode. | UI encoder behavior may overlap with screen controls; coordinate ownership. |
| EXTITask / StartEXTITask | Handles button/switch/fault GPIO events with debounce. | UI button event maps mode cycling and motor re-arm behavior. |
| CommTask | Placeholder. | Candidate for future external command API. |
| StorageTask | Placeholder. | Candidate for settings persistence. |
| StepperTask | Recommended but not confirmed in current freertos.c. | Should call TMC2130Motion_Task() every 1 ms if stepper motion layer is used. |

# 4. BLDC motor API: motor_foc.h

The BLDC API exposes a command/request model. UI code can safely request mode/setpoint/state changes without directly editing live control variables.

## 4.1 Enums

typedef enum
{
  MOTOR_FOC_STATE_IDLE = 0,
  MOTOR_FOC_STATE_CALIB,
  MOTOR_FOC_STATE_ALIGN,
  MOTOR_FOC_STATE_RUN,
  MOTOR_FOC_STATE_FAULT,
} MotorFOC_State_t;

typedef enum
{
  MOTOR_FOC_MODE_TORQUE = 0,
  MOTOR_FOC_MODE_SPEED,
  MOTOR_FOC_MODE_POSITION,
} MotorFOC_Mode_t;

| Value | Display meaning | UI action |
| --- | --- | --- |
| IDLE | Motor controller is not running output control. | Show idle/safe state. |
| CALIB | Current offset calibration active. | Disable motion commands until complete. |
| ALIGN | Encoder/electrical alignment active. | Show preparing/alignment state. |
| RUN | Motor control loop active. | Enable normal setpoint controls. |
| FAULT | Fault state. | Show fault, disable new motion until cleared/handled. |

## 4.2 Lifecycle and request APIs

| Function | UI use | Notes |
| --- | --- | --- |
| void MotorFOC_Init(void); | No | Lifecycle/owner task only. |
| void MotorFOC_Start(void); | No | Lifecycle/owner task only. Starts hardware and calibration. |
| void MotorFOC_Stop(void); | Maybe via service | Queues controlled peripheral shutdown. Prefer wrapping in UI_BldcStop(). |
| void MotorFOC_Arm(void); | Maybe via service | Queues arm/alignment. Prefer MotorFOC_RequestArm() or UI_BldcArm(). |
| void MotorFOC_Disarm(void); | Maybe via service | Queues disarm. Prefer MotorFOC_RequestDisarm() or UI_BldcDisarm(). |
| void MotorFOC_RequestArm(void); | Yes | Request driver enable + alignment through owner loop. |
| void MotorFOC_RequestDisarm(void); | Yes | Request driver disable. |
| void MotorFOC_RequestEmergencyStop(void); | Yes | Immediate safety UI action. |
| void MotorFOC_RequestZeroEncoder(void); | Yes, with caution | Zero BLDC encoder reference. Use only from calibration/setup UI. |
| void MotorFOC_RequestStop(void); | Yes | Request controlled stop/shutdown. |

## 4.3 Mode and setpoint APIs

| Function | Purpose | UI notes |
| --- | --- | --- |
| void MotorFOC_SetMode(MotorFOC_Mode_t mode); | Request torque/speed/position mode. | Use enum values; invalid values ignored. |
| void MotorFOC_SetCommand(MotorFOC_Mode_t mode, float speed_rpm, float torque_iq_A, int32_t position_counts); | Atomic combined mode/setpoint update. | Best API when UI changes multiple fields at once. |
| void MotorFOC_SetSpeedRPM(float rpm); | Set speed reference in RPM. | Used in SPEED mode. |
| void MotorFOC_SetTorqueCurrentA(float iq_A); | Set torque/q-axis current reference in amps. | Used in TORQUE mode. Clamp is applied internally. |
| void MotorFOC_SetPositionCounts(int32_t pos_counts); | Set absolute position in encoder counts. | Used in POSITION mode. |
| void MotorFOC_SetPositionRevolutions(float revolutions); | Set absolute position in mechanical revolutions. | Converted by MOTOR_FOC_ENCODER_CPR. |

## 4.4 BLDC getters for UI display

| Function | Returns | Recommended UI label |
| --- | --- | --- |
| MotorFOC_State_t MotorFOC_GetState(void); | Applied controller state. | State |
| MotorFOC_Mode_t MotorFOC_GetMode(void); | Applied mode. | Applied mode |
| MotorFOC_Mode_t MotorFOC_GetRequestedMode(void); | Requested command mode. | Requested mode |
| float MotorFOC_GetIa(void); | Phase A current, A. | Ia |
| float MotorFOC_GetIb(void); | Phase B current, A. | Ib |
| float MotorFOC_GetIc(void); | Phase C current, A. | Ic |
| float MotorFOC_GetId(void); | d-axis current, A. | Id |
| float MotorFOC_GetIq(void); | q-axis current, A. | Iq |
| float MotorFOC_GetSpeedRPM(void); | Estimated speed, RPM. | Speed |
| int32_t MotorFOC_GetPositionCounts(void); | TIM2 encoder count. | Position counts |

## 4.5 BLDC UI examples

/* Arm and run speed mode */
MotorFOC_SetMode(MOTOR_FOC_MODE_SPEED);
MotorFOC_SetSpeedRPM(500.0f);
MotorFOC_RequestArm();

/* Position mode */
MotorFOC_SetCommand(MOTOR_FOC_MODE_POSITION,
                    0.0f,      /* speed rpm unused here */
                    0.0f,      /* torque current unused here */
                    4000);     /* one rev if CPR = 4000 */

/* Emergency stop */
MotorFOC_RequestEmergencyStop();

# 5. TMC2130 low-level driver API: tmc2130.h

Use this layer for initialization, diagnostics, and bring-up. For normal UI motion commands, prefer the TMC2130Motion API in Section 6.

## 5.1 Hardware mapping

| Signal | MCU / project define | UI relevance |
| --- | --- | --- |
| CSN | EXT_NSS / PA4 / GPIO software CS | Driver-controlled only. |
| SCK | EXT_SCK / PC10 / SPI3_SCK | Driver-controlled only. |
| SDO | EXT_MISO / PC11 / SPI3_MISO | Driver-controlled only. |
| SDI | EXT_MOSI / PC12 / SPI3_MOSI | Driver-controlled only. |
| STEP | STEP / PH9 / TIM12_CH2 | Motion layer controls. |
| DIR | DIR / PH7 | Direction control. |
| ENN | DRV_ENN / PG3 active-low | Enable/disable stepper driver. |
| DIAG0 / DIAG1 | PC7 / PC8 | Fault/status display. |
| DC_IN | PG2 | dcStep status/input. |

## 5.2 Types

typedef enum
{
    TMC2130_DIR_FORWARD = 0,
    TMC2130_DIR_REVERSE = 1,
} TMC2130_Direction_t;

typedef struct
{
    SPI_HandleTypeDef *hspi;
    GPIO_TypeDef *cs_port;
    uint16_t cs_pin;
    TIM_HandleTypeDef *step_tim;
    uint32_t step_channel;
    uint32_t spi_timeout_ms;
    uint32_t step_timer_clock_hz;
    bool step_clock_running;
    uint32_t step_frequency_hz;
} TMC2130_Handle_t;

typedef struct
{
    uint8_t ihold;
    uint8_t irun;
    uint8_t ihold_delay;
    uint16_t microsteps;
    bool interpolate_256;
    bool stealthchop_enable;
    uint8_t toff;
    uint8_t hstrt;
    uint8_t hend;
    uint8_t tbl;
    uint32_t tpwmthrs;
    uint32_t tcoolthrs;
    uint32_t thigh;
    uint32_t pwmconf;
    bool dcstep_enable;
} TMC2130_Config_t;

## 5.3 Functions

| Function | Purpose | UI notes |
| --- | --- | --- |
| void TMC2130_ConfigDefaults(TMC2130_Config_t *cfg); | Load conservative default driver config. | Setup only. |
| HAL_StatusTypeDef TMC2130_Init(...); | Initialize driver with board default CS pin. | Setup only. |
| HAL_StatusTypeDef TMC2130_InitEx(...); | Initialize with explicit CS pin. | Setup/advanced. |
| HAL_StatusTypeDef TMC2130_WriteReg(...); | Write raw TMC2130 register. | Debug/advanced only. |
| HAL_StatusTypeDef TMC2130_ReadReg(...); | Read raw TMC2130 register. | Debug/status. |
| HAL_StatusTypeDef TMC2130_Enable(TMC2130_Handle_t *dev, bool enable); | Enable/disable active-low ENN. | Usually called via motion layer. |
| void TMC2130_SetDirection(TMC2130_Direction_t dir); | Set DIR pin. | Motion layer normally owns this. |
| HAL_StatusTypeDef TMC2130_SetStepFrequency(...); | Set STEP PWM frequency. | Low-level; motion layer uses this. |
| HAL_StatusTypeDef TMC2130_StartStepClock(...); | Start continuous STEP clock. | Avoid from UI screens; use motion layer. |
| HAL_StatusTypeDef TMC2130_StopStepClock(...); | Stop STEP clock. | Avoid from UI screens; use motion layer. |
| bool TMC2130_IsStepClockRunning(...); | Return whether STEP clock is running. | Display/debug. |
| uint32_t TMC2130_GetStepFrequencyHz(...); | Return current STEP frequency. | Display/debug. |
| HAL_StatusTypeDef TMC2130_MoveBlocking(...); | Blocking timed move. | Do not call from UI thread. |
| HAL_StatusTypeDef TMC2130_ReadGSTAT(...); | Read global status. | Fault display/debug. |
| HAL_StatusTypeDef TMC2130_ClearGSTAT(...); | Clear global status bits. | Service/maintenance action. |
| HAL_StatusTypeDef TMC2130_ReadDRVStatus(...); | Read driver status register. | Fault/status display/debug. |
| bool TMC2130_ReadDiag0(void); | Read DIAG0 GPIO. | Display/fault input. |
| bool TMC2130_ReadDiag1(void); | Read DIAG1 GPIO. | Display/fault input. |
| bool TMC2130_ReadDcIn(void); | Read DC_IN GPIO. | dcStep status/input. |

# 6. Stepper motion API: tmc2130_motion.h

The motion layer is the recommended UI-facing stepper control surface. It is non-blocking and acceleration-limited. A periodic task must call TMC2130Motion_Task().

## 6.1 Motion states and limits

typedef enum
{
    TMC2130_MOTION_IDLE = 0,
    TMC2130_MOTION_MOVING,
    TMC2130_MOTION_SYNC,
    TMC2130_MOTION_STOPPING,
    TMC2130_MOTION_FAULT
} TMC2130Motion_State_t;

typedef struct
{
    float max_speed_steps_s;
    float accel_steps_s2;
    float min_run_speed_steps_s;
    float position_tolerance_steps;
} TMC2130Motion_Limits_t;

| State | Meaning | UI behavior |
| --- | --- | --- |
| IDLE | No active motion. | Enable move commands. |
| MOVING | Executing move-to/move-by profile. | Show active motion; enable stop. |
| SYNC | Following BLDC encoder-derived target. | Show sync active; enable sync disable/stop. |
| STOPPING | Controlled deceleration to stop. | Disable new commands until idle or allow emergency stop. |
| FAULT | Motion output failed or configuration invalid. | Show fault and require service/reset. |

## 6.2 Motion functions

| Function | Purpose | UI notes |
| --- | --- | --- |
| void TMC2130Motion_DefaultLimits(TMC2130Motion_Limits_t *limits); | Fill default speed/acceleration/tolerance. | Setup. |
| HAL_StatusTypeDef TMC2130Motion_Init(...); | Initialize motion handle with driver and limits. | Setup. |
| HAL_StatusTypeDef TMC2130Motion_Enable(..., bool enable); | Enable/disable stepper driver through motion layer. | UI stepper enable toggle. |
| HAL_StatusTypeDef TMC2130Motion_MoveTo(..., float target_steps); | Move to absolute estimated step position. | UI absolute move. |
| HAL_StatusTypeDef TMC2130Motion_MoveBy(..., float delta_steps); | Move relative by delta steps. | UI jog/move command. |
| HAL_StatusTypeDef TMC2130Motion_Stop(...); | Controlled deceleration stop. | Normal stop button. |
| HAL_StatusTypeDef TMC2130Motion_EmergencyStop(...); | Immediate stop output. | Emergency UI action. |
| void TMC2130Motion_Task(..., float dt_s); | Periodic motion update. | Owner task only, not UI callback. |
| HAL_StatusTypeDef TMC2130Motion_EnableBldcSync(...); | Enable BLDC-follow sync. | UI sync enable. |
| void TMC2130Motion_DisableBldcSync(...); | Disable BLDC-follow sync. | UI sync disable. |
| TMC2130Motion_State_t TMC2130Motion_GetState(...); | Get motion state. | Display. |
| float TMC2130Motion_GetPositionSteps(...); | Get estimated stepper position. | Display. |
| float TMC2130Motion_GetTargetSteps(...); | Get current target. | Display. |
| float TMC2130Motion_GetVelocityStepsS(...); | Get current commanded velocity. | Display. |

## 6.3 BLDC-stepper synchronization

Sync mode updates the stepper target from BLDC encoder counts each motion tick:

target_steps = sync_offset_steps
             + MotorFOC_GetPositionCounts() * sync_scale_steps_per_bldc_count;

/* Example: 200 full steps/rev, 16 microsteps, 1:1 ratio, BLDC CPR = 4000 */
float scale = (200.0f * 16.0f * 1.0f) / (float)MOTOR_FOC_ENCODER_CPR;
TMC2130Motion_EnableBldcSync(&g_stepper_motion, scale, 0.0f);

## 6.4 Stepper UI examples

/* Enable stepper */
TMC2130Motion_Enable(&g_stepper_motion, true);

/* Jog forward by 1000 microsteps */
TMC2130Motion_MoveBy(&g_stepper_motion, 1000.0f);

/* Move to absolute estimated position */
TMC2130Motion_MoveTo(&g_stepper_motion, 3200.0f);

/* Normal stop */
TMC2130Motion_Stop(&g_stepper_motion);

/* Emergency stop */
TMC2130Motion_EmergencyStop(&g_stepper_motion);

/* Enable BLDC sync */
TMC2130Motion_EnableBldcSync(&g_stepper_motion, scale, offset_steps);

/* Disable sync */
TMC2130Motion_DisableBldcSync(&g_stepper_motion);

# 7. Telemetry, buzzer, LEDs, TouchGFX

## 7.1 Telemetry API

| Function | Purpose | UI notes |
| --- | --- | --- |
| void Telemetry_Init(void); | Initialize debug framebuffer telemetry overlay. | Debug only; avoid in final TouchGFX UI unless gated. |
| void Telemetry_Update(void); | Draw/update overlay. | Runs in GUI task currently. |
| void Telemetry_SetFrameBufferAddress(uint32_t fb_addr); | Set LTDC framebuffer address. | Low-level/debug; not for screen callbacks. |

## 7.2 Buzzer API

| Function | Purpose | UI notes |
| --- | --- | --- |
| void Buzzer_Init(Buzzer_Handle_t *bz, TIM_HandleTypeDef *htim8); | Initialize buzzer handle. | Setup. |
| HAL_StatusTypeDef Buzzer_ToneHz(...); | Start/retune tone. | Can be exposed through UI service. |
| HAL_StatusTypeDef Buzzer_Stop(...); | Stop buzzer. | Safe UI command. |
| HAL_StatusTypeDef Buzzer_Beep(...); | Blocking beep using HAL_Delay(). | Do not call from UI thread. |

## 7.3 LEDs API

| Function | Purpose |
| --- | --- |
| LED0_On / LED0_Off / LED0_Toggle | Control LED0. |
| LED1_On / LED1_Off / LED1_Toggle | Control LED1. |
| LED2_On / LED2_Off / LED2_Toggle | Control LED2. |
| LEDs_Init | Declared, but GPIO init is generated by CubeMX; confirm implementation before use. |

## 7.4 TouchGFX lifecycle API

| Function | Purpose | UI notes |
| --- | --- | --- |
| void MX_TouchGFX_PreOSInit(void); | Generated pre-OS init hook. | System init only. |
| void MX_TouchGFX_Init(void); | Initialize TouchGFX. | System init only. |
| void MX_TouchGFX_Process(void); | Run TouchGFX task entry. | Currently called by GUI task. |
| void TouchGFX_Task(void* argument); | Generated task entry wrapper. | Alternative generated task API. |

# 8. Recommended UI service facade

The current project exposes several module APIs. For production UI, create a single facade that screens call. This keeps TouchGFX code independent of motor/driver internals.

typedef struct {
    MotorFOC_State_t bldc_state;
    MotorFOC_Mode_t bldc_mode;
    MotorFOC_Mode_t bldc_requested_mode;
    float bldc_speed_rpm;
    int32_t bldc_position_counts;
    float ia, ib, ic, id, iq;

    TMC2130Motion_State_t stepper_state;
    float stepper_position_steps;
    float stepper_target_steps;
    float stepper_velocity_steps_s;

    bool stepper_diag0;
    bool stepper_diag1;
    bool stepper_dc_in;
    uint32_t stepper_gstat;
    uint32_t stepper_drv_status;

    uint32_t faults;
} UI_SystemSnapshot_t;

void UI_GetSnapshot(UI_SystemSnapshot_t *out);
void UI_BldcSetMode(MotorFOC_Mode_t mode);
void UI_BldcSetSpeedRPM(float rpm);
void UI_BldcSetTorqueA(float iq_a);
void UI_BldcSetPositionRev(float rev);
void UI_BldcArm(void);
void UI_BldcDisarm(void);
void UI_BldcEmergencyStop(void);

void UI_StepperEnable(bool enable);
void UI_StepperMoveTo(float steps);
void UI_StepperMoveBy(float steps);
void UI_StepperStop(void);
void UI_StepperEmergencyStop(void);
void UI_StepperSyncEnable(float scale, float offset);
void UI_StepperSyncDisable(void);

# 9. Globals and variables

| Symbol | Where | UI usage |
| --- | --- | --- |
| MotorTaskHandle, GUITaskHandle, HIDTaskHandle, CommTaskHandle, StorageTaskHandle, EXTITaskHandle | freertos.c | Internal RTOS handles. Do not use from screens. |
| HIDQHandle | freertos.c | Internal queue handle. Not currently a public UI API. |
| ped_enc | freertos.c | Internal UI encoder value. Use a service getter if UI needs it. |
| g_tmc2130, g_stepper_motion | Recommended static globals if integrated | Should be owned by a service/task module, not TouchGFX screens. |

# 10. UI developer checklist

Use the BLDC request/setter APIs; do not access TIM1/TIM2/ADC directly.

Use TMC2130Motion APIs for stepper movement; avoid direct STEP clock calls from UI.

Do not call blocking functions from TouchGFX event handlers.

Display state through getters or UI_GetSnapshot().

Show BLDC state, requested/applied mode, speed RPM, position counts, and current values.

Show stepper state, estimated position, target, velocity, and sync state.

Expose emergency stop as a top-level UI action.

Gate debug telemetry overlay or remove it from production screens.

Coordinate UI encoder behavior with TouchGFX screen controls; avoid competing input ownership.

# 11. Source files used

| File | Used for |
| --- | --- |
| Core/Inc/motor_foc.h | BLDC public API, modes, states, constants. |
| Core/Inc/tmc2130.h | TMC2130 low-level API, config, handle, status functions. |
| Core/Inc/tmc2130_motion.h | Stepper acceleration/sync API. |
| Core/Inc/telemetry.h | Telemetry overlay API. |
| Core/Inc/buzzer.h | Buzzer API. |
| Core/Inc/leds.h | LED API. |
| TouchGFX/App/app_touchgfx.h | TouchGFX lifecycle API. |
| Core/Src/freertos.c | Runtime task ownership and current UI encoder behavior. |