FRW4_0_RTOS UI Developer API Guide

BLDC control, TMC2130 stepper control, telemetry, UI events, and safety-facing APIs

Generated from the latest repository state inspected on main branch

| Area | UI developer summary |
| --- | --- |
| BLDC motor control | Use MotorFOC_SetMode(), MotorFOC_SetSpeedRPM(), MotorFOC_SetTorqueCurrentA(), MotorFOC_SetPositionRevolutions()/Counts(), request APIs, and getters. Avoid calling lifecycle functions except through the motor owner/service layer. |
| TMC2130 stepper low-level driver | Use only for initialization, enable/disable, raw register read/write, direct STEP clock testing, and diagnostics. Prefer the motion layer for UI actions. |
| TMC2130 motion layer | Recommended UI-facing stepper API. Provides non-blocking move-to/move-by, acceleration, stop, emergency stop, and BLDC position-follow sync. |
| Telemetry | Use getters from MotorFOC and TMC2130Motion for UI widgets. Telemetry_Update() is debug/framebuffer-specific and should not be mixed blindly with TouchGFX rendering. |
| TouchGFX | MX_TouchGFX_Process() is already called by GUITask. UI screens should call application service APIs, not low-level HAL directly. |

# 1. Source files covered

This guide covers the UI-facing APIs found in these project files:

| File | Why UI developers care |
| --- | --- |
| Core/Inc/motor_foc.h | BLDC motor state, mode, commands, requests, and telemetry getters |
| Core/Inc/tmc2130.h | Low-level TMC2130 driver and direct STEP/DIR/enable API |
| Core/Inc/tmc2130_motion.h | High-level acceleration-limited non-blocking stepper motion API |
| Core/Inc/telemetry.h | Debug framebuffer telemetry overlay API |
| Core/Inc/buzzer.h | Buzzer tone/beep API |
| Core/Inc/leds.h | LED utility API |
| TouchGFX/App/app_touchgfx.h | TouchGFX lifecycle functions |
| Core/Src/freertos.c | Task topology, UI encoder helper behavior, event routing |

# 2. Context and ownership rules

| Rule | Reason |
| --- | --- |
| Do not write HAL timers/GPIO/FOC internals directly from UI screens. | The motor and stepper modules own their hardware state. UI should call request/setpoint APIs. |
| Do not call blocking motion APIs from the TouchGFX GUI loop. | Blocking calls can stall rendering and input handling. Use non-blocking motion APIs and periodic tasks. |
| Use request-style motor APIs for arm/disarm/stop. | The BLDC state machine is applied by the motor owner loop. |
| Use TMC2130Motion_* for stepper moves in UI logic. | It provides acceleration/deceleration and BLDC synchronization. |
| Use getters for display values; do not share global variables. | Getters provide a stable API boundary and avoid coupling UI to task internals. |

# 3. Runtime task map

| Task | Responsibility |
| --- | --- |
| MotorTask | Runs MotorFOC_Init(), MotorFOC_Start(), then MotorFOC() every 1 ms using vTaskDelayUntil(). Owns BLDC slow-loop execution. |
| GUITask | Runs TouchGFX processing and debug telemetry update. UI code should normally live in TouchGFX model/presenter/view, not in this task body. |
| HIDTask | Reads UI encoder and updates requested BLDC setpoints based on requested mode. |
| EXTITask | Debounces button/EXTI events and queues BLDC requests such as disarm, zero encoder, and arm. |
| CommTask | Placeholder for communication service. |
| StorageTask | Placeholder for storage service. |

# 4. BLDC motor API: motor_foc.h

The BLDC API is split into lifecycle functions, UI-safe setpoint/request APIs, and telemetry getters. For UI development, prefer the setpoint, request, and getter APIs.

| Enum | Values | UI meaning |
| --- | --- | --- |
| MotorFOC_State_t | IDLE, CALIB, ALIGN, RUN, FAULT | Current BLDC state. Display as status and disable unsafe UI actions when FAULT/CALIB/ALIGN. |
| MotorFOC_Mode_t | TORQUE, SPEED, POSITION | Selected control mode for BLDC setpoint interpretation. |

## 4.1 UI-safe BLDC commands

These functions are intended for UI input handlers, sliders, mode buttons, and service-layer calls.

| Function | Purpose | UI notes |
| --- | --- | --- |
| void MotorFOC_SetMode(MotorFOC_Mode_t mode) | Requests Torque, Speed, or Position mode. | Use when user changes mode. Mode is applied by motor loop; use MotorFOC_GetRequestedMode() for current requested UI selection. |
| void MotorFOC_SetCommand(MotorFOC_Mode_t mode, float speed_rpm, float torque_iq_A, int32_t position_counts) | Atomic-style combined mode and setpoint update. | Best for UI screens that update several setpoints together. |
| void MotorFOC_SetSpeedRPM(float rpm) | Sets speed reference in RPM. | Only meaningful in SPEED mode. Value is clamped by motor config. |
| void MotorFOC_SetTorqueCurrentA(float iq_A) | Sets q-axis torque current reference in amps. | Only meaningful in TORQUE mode. Keep UI range conservative. |
| void MotorFOC_SetPositionCounts(int32_t pos_counts) | Sets absolute encoder-count position target. | Use for raw position control. Counts are TIM2 quadrature counts. |
| void MotorFOC_SetPositionRevolutions(float revolutions) | Sets absolute mechanical position target in revolutions. | Convenient for UI position widgets. |

## 4.2 BLDC request APIs

These queue state-machine requests. The motor owner loop applies them.

| Function | Purpose | UI notes |
| --- | --- | --- |
| void MotorFOC_RequestArm(void) | Queue driver enable and alignment. | Use only when UI explicitly starts motor system. |
| void MotorFOC_RequestDisarm(void) | Queue driver disable and safe zero vector. | Use for normal stop/disable. |
| void MotorFOC_RequestEmergencyStop(void) | Queue immediate fault-style stop. | Use for emergency UI control. Should also disable related UI actions. |
| void MotorFOC_RequestZeroEncoder(void) | Queue BLDC encoder counter reset. | Use only after controlled stop/alignment workflow. |
| void MotorFOC_RequestStop(void) | Queue controlled peripheral shutdown. | Usually not a UI button; service/shutdown action. |
| void MotorFOC_Arm(void) | Wrapper for RequestArm. | Safe but prefer explicit RequestArm for clarity. |
| void MotorFOC_Disarm(void) | Wrapper for RequestDisarm. | Safe but prefer explicit RequestDisarm for clarity. |
| void MotorFOC_Stop(void) | Wrapper for RequestStop. | Do not call from rendering loop. |

## 4.3 BLDC status and telemetry getters

Use these for UI widgets and debug panels.

| Function | Returns | Display/use |
| --- | --- | --- |
| MotorFOC_State_t MotorFOC_GetState(void) | Applied state | Status badge: Idle / Calib / Align / Run / Fault. |
| MotorFOC_Mode_t MotorFOC_GetMode(void) | Applied mode | Shows what the motor loop is currently using. |
| MotorFOC_Mode_t MotorFOC_GetRequestedMode(void) | Requested mode | Use to drive UI mode selection state. |
| float MotorFOC_GetIa/Ib/Ic(void) | Phase currents in amps | Debug/current panel. |
| float MotorFOC_GetId/Iq(void) | d/q currents in amps | FOC tuning/debug panel. |
| float MotorFOC_GetSpeedRPM(void) | Estimated speed RPM | Speed display and stop wait logic. |
| int32_t MotorFOC_GetPositionCounts(void) | TIM2 encoder counts | Position display and BLDC-sync input. |

## 4.4 Owner-only BLDC lifecycle

These are not normal UI callbacks. They are used by MotorTask/startup.

| Function | Purpose | UI rule |
| --- | --- | --- |
| void MotorFOC_Init(void) | Initialize internal BLDC state. | Owner-only: startup task. |
| void MotorFOC_Start(void) | Start hardware timers/ADCs/calibration. | Owner-only: startup task. |
| void MotorFOC(void) | 1 ms slow-loop state machine. | Owner-only: MotorTask periodic loop. |
| void Motor_OpenLoopStep_20kHz(void) | Debug open-loop step function. | Do not expose to UI without engineering debug mode. |

# 5. TMC2130 low-level driver API: tmc2130.h

The low-level TMC2130 driver configures U10 and provides direct register access, enable, direction, STEP clock, and diagnostics. For ordinary UI motion, prefer the higher-level TMC2130Motion API in Section 6.

| Type | Fields / values | Notes |
| --- | --- | --- |
| TMC2130_Direction_t | TMC2130_DIR_FORWARD, TMC2130_DIR_REVERSE | Used by direct STEP clock and blocking moves. |
| TMC2130_Register_t | GCONF, GSTAT, IOIN, IHOLD_IRUN, CHOPCONF, DRV_STATUS, PWMCONF, etc. | Raw register addresses for engineering diagnostics. |
| TMC2130_Handle_t | hspi, cs_port, step_tim, step_channel, spi_timeout_ms, step_clock_running, step_frequency_hz | Driver instance. UI should not edit fields directly. |
| TMC2130_Config_t | ihold, irun, microsteps, stealthchop_enable, chopper params, thresholds, dcstep_enable | Startup configuration. Usually fixed by firmware, not a normal UI runtime object. |

## 5.1 TMC2130 setup and raw register API

| Function | Purpose | UI notes |
| --- | --- | --- |
| void TMC2130_ConfigDefaults(TMC2130_Config_t *cfg) | Fill safe default TMC config. | Use during service init, not UI event. |
| HAL_StatusTypeDef TMC2130_Init(...) | Initialize board-default TMC2130 using SPI3, EXT_NSS GPIO CS, TIM12 STEP. | Owner/service init. |
| HAL_StatusTypeDef TMC2130_InitEx(...) | Initialize with explicit chip-select pin. | Engineering/testing use. |
| HAL_StatusTypeDef TMC2130_WriteReg(..., uint8_t addr, uint32_t value) | Write raw 32-bit register. | Engineering/debug only. Validate before exposing. |
| HAL_StatusTypeDef TMC2130_ReadReg(..., uint8_t addr, uint32_t *value) | Read raw 32-bit register. | Engineering/debug status panels only. |

## 5.2 Direct stepper control

| Function | Purpose | UI notes |
| --- | --- | --- |
| HAL_StatusTypeDef TMC2130_Enable(TMC2130_Handle_t *dev, bool enable) | Enable/disable driver via DRV_ENN. | Prefer motion-layer enable. |
| void TMC2130_SetDirection(TMC2130_Direction_t dir) | Set DIR pin. | Prefer MoveTo/MoveBy unless manual jog mode. |
| HAL_StatusTypeDef TMC2130_SetStepFrequency(..., uint32_t frequency_hz) | Configure TIM12 STEP frequency. | Low-level. Motion layer changes frequency automatically. |
| HAL_StatusTypeDef TMC2130_StartStepClock(..., uint32_t frequency_hz) | Start continuous STEP PWM. | Manual jog/testing only. |
| HAL_StatusTypeDef TMC2130_StopStepClock(TMC2130_Handle_t *dev) | Stop STEP PWM. | Manual jog/testing only. |
| bool TMC2130_IsStepClockRunning(const TMC2130_Handle_t *dev) | Returns whether STEP clock is running. | Status/debug widget. |
| uint32_t TMC2130_GetStepFrequencyHz(const TMC2130_Handle_t *dev) | Current requested STEP frequency. | Status/debug widget. |
| HAL_StatusTypeDef TMC2130_MoveBlocking(..., int32_t steps, uint32_t frequency_hz) | Simple blocking timed move. | Do not call from GUI thread. Use TMC2130Motion for UI. |

## 5.3 TMC2130 diagnostics

| Function | Purpose | UI notes |
| --- | --- | --- |
| HAL_StatusTypeDef TMC2130_ReadGSTAT(..., uint32_t *gstat) | Read global status register. | Show reset/driver/charge-pump flags in debug page. |
| HAL_StatusTypeDef TMC2130_ClearGSTAT(TMC2130_Handle_t *dev) | Clear GSTAT latched flags. | Engineering/debug action. |
| HAL_StatusTypeDef TMC2130_ReadDRVStatus(..., uint32_t *drv_status) | Read driver status register. | Debug/fault page. |
| bool TMC2130_ReadDiag0(void) | Read DIAG0 pin. | Status indicator. |
| bool TMC2130_ReadDiag1(void) | Read DIAG1 pin. | Status indicator. |
| bool TMC2130_ReadDcIn(void) | Read DC_IN pin. | dcStep/debug indicator. |

# 6. TMC2130 acceleration and BLDC sync API: tmc2130_motion.h

This is the preferred UI-facing stepper motion layer. It is non-blocking and should be advanced periodically by a task, usually at 1 ms. It maintains estimated stepper position and can follow BLDC encoder position.

| Type | Fields / values | UI meaning |
| --- | --- | --- |
| TMC2130Motion_State_t | IDLE, MOVING, SYNC, STOPPING, FAULT | Stepper motion status badge. |
| TMC2130Motion_Limits_t | max_speed_steps_s, accel_steps_s2, min_run_speed_steps_s, position_tolerance_steps | Motion tuning. Usually firmware/service config. |
| TMC2130Motion_Handle_t | driver, limits, state, position_steps, target_steps, velocity_steps_s, enabled, sync_enabled, sync scale/offset | Motion controller state. UI should read through getters. |

## 6.1 Motion setup

| Function | Purpose | UI notes |
| --- | --- | --- |
| void TMC2130Motion_DefaultLimits(TMC2130Motion_Limits_t *limits) | Fill default motion limits. | Service init. |
| HAL_StatusTypeDef TMC2130Motion_Init(..., TMC2130_Handle_t *driver, const TMC2130Motion_Limits_t *limits) | Initialize motion controller using low-level driver. | Service init. |
| HAL_StatusTypeDef TMC2130Motion_Enable(TMC2130Motion_Handle_t *motion, bool enable) | Enable/disable stepper motion/driver. | Safe UI start/enable toggle if called through service. |

## 6.2 Non-blocking stepper motion

| Function | Purpose | UI notes |
| --- | --- | --- |
| HAL_StatusTypeDef TMC2130Motion_MoveTo(..., float target_steps) | Move to absolute estimated position in steps/microsteps. | Use for absolute stepper target from UI. |
| HAL_StatusTypeDef TMC2130Motion_MoveBy(..., float delta_steps) | Move relative by delta steps. | Use for jog buttons or relative move UI. |
| HAL_StatusTypeDef TMC2130Motion_Stop(TMC2130Motion_Handle_t *motion) | Controlled deceleration stop. | Normal stop button. |
| HAL_StatusTypeDef TMC2130Motion_EmergencyStop(TMC2130Motion_Handle_t *motion) | Immediate stop/disable. | Emergency stop button. |
| void TMC2130Motion_Task(TMC2130Motion_Handle_t *motion, float dt_s) | Periodic motion update. | Owner task only. Do not call from drawing code. |

## 6.3 BLDC synchronization

| Function | Purpose | UI notes |
| --- | --- | --- |
| HAL_StatusTypeDef TMC2130Motion_EnableBldcSync(..., float scale_steps_per_bldc_count, float offset_steps) | Enable stepper target following BLDC encoder count. | Use for linked motion. UI can expose ratio and offset if needed. |
| void TMC2130Motion_DisableBldcSync(TMC2130Motion_Handle_t *motion) | Return to standalone stepper control. | Disable sync mode. |

## 6.4 Motion telemetry getters

| Function | Returns | Display/use |
| --- | --- | --- |
| TMC2130Motion_State_t TMC2130Motion_GetState(const TMC2130Motion_Handle_t *motion) | Motion state | Stepper status badge. |
| float TMC2130Motion_GetPositionSteps(const TMC2130Motion_Handle_t *motion) | Estimated stepper position | Stepper position display. |
| float TMC2130Motion_GetTargetSteps(const TMC2130Motion_Handle_t *motion) | Current target | Target display. |
| float TMC2130Motion_GetVelocityStepsS(const TMC2130Motion_Handle_t *motion) | Current commanded velocity | Speed/velocity display. |

# 7. UI support APIs: telemetry, buzzer, LEDs, TouchGFX

## 7.1 Telemetry overlay

Debug framebuffer telemetry can conflict with TouchGFX rendering if both write the same layer. Prefer TouchGFX widgets for production UI.

| Function | Purpose | UI notes |
| --- | --- | --- |
| void Telemetry_Init(void) | Initialize direct framebuffer telemetry. | Debug only unless coordinated with TouchGFX. |
| void Telemetry_Update(void) | Draw/update debug overlay. | Currently called by GUITask every 20 ms. |
| void Telemetry_SetFrameBufferAddress(uint32_t fb_addr) | Set framebuffer address for telemetry overlay. | Engineering/debug use. |

## 7.2 Buzzer

| Function | Purpose | UI notes |
| --- | --- | --- |
| void Buzzer_Init(Buzzer_Handle_t *bz, TIM_HandleTypeDef *htim8) | Initialize buzzer handle for TIM8 CH1. | Service init. |
| HAL_StatusTypeDef Buzzer_ToneHz(..., uint32_t freq_hz, uint8_t duty_percent) | Start or retune tone. | Non-blocking tone start; stop separately. |
| HAL_StatusTypeDef Buzzer_Stop(Buzzer_Handle_t *bz) | Stop buzzer. | Safe UI feedback action. |
| HAL_StatusTypeDef Buzzer_Beep(..., uint32_t freq_hz, uint32_t duration_ms, uint8_t duty_percent) | Blocking beep using HAL_Delay. | Avoid in GUI thread; use service task or non-blocking tone + timer. |

## 7.3 LEDs

| Function | Purpose | UI notes |
| --- | --- | --- |
| LED0_On/Off/Toggle() | Control LED0. | Debug/status indicator. |
| LED1_On/Off/Toggle() | Control LED1. | Debug/status indicator. |
| LED2_On/Off/Toggle() | Control LED2. | Debug/status indicator. |
| void LEDs_Init(void) | Declared but current LEDs are CubeMX GPIO-initialized. | Do not rely on it unless implementation exists and is wired. |

## 7.4 TouchGFX lifecycle

| Function | Purpose | UI notes |
| --- | --- | --- |
| void MX_TouchGFX_PreOSInit(void) | Pre-OS TouchGFX hook. | Startup only. |
| void MX_TouchGFX_Init(void) | Initialize TouchGFX. | Startup only. |
| void MX_TouchGFX_Process(void) | Run TouchGFX event loop/task entry. | Already called by GUITask. Do not call from random code. |
| void TouchGFX_Task(void *argument) | Generated TouchGFX task entry. | Generated interface; normally not called directly by UI code. |

# 8. Globals and handles visible in current code

The following symbols exist in the firmware, but UI code should not directly mutate them. Prefer the API functions above.

| Symbol | Location | UI guidance |
| --- | --- | --- |
| float ped_enc | Core/Src/freertos.c | Internal HID/UI encoder value. Do not use as stable public API. |
| MotorTaskHandle, GUITaskHandle, HIDTaskHandle, CommTaskHandle, StorageTaskHandle, EXTITaskHandle | Core/Src/freertos.c | Task handles. Do not use from UI screens except debug introspection. |
| HIDQHandle | Core/Src/freertos.c | Message queue handle currently created. Treat as internal until a formal HID message API is defined. |
| hspi3, htim12, htim8, htim3, etc. | Generated HAL files | Hardware handles. Service-layer initialization only; not UI widget code. |
| TMC2130_Handle_t / TMC2130Motion_Handle_t instances | Application/service layer | Should be wrapped by application service functions before UI calls. |

# 9. Recommended UI workflows

## 9.1 Select BLDC control mode

// Mode button callback
MotorFOC_SetMode(MOTOR_FOC_MODE_SPEED);      // or TORQUE / POSITION

// UI status refresh
MotorFOC_Mode_t requested = MotorFOC_GetRequestedMode();
MotorFOC_State_t state = MotorFOC_GetState();

## 9.2 Speed slider

// User moves speed slider
float rpm = ui_slider_value_rpm;
MotorFOC_SetSpeedRPM(rpm);

// Display
float actual_rpm = MotorFOC_GetSpeedRPM();

## 9.3 Torque slider

// User moves torque slider
float iq_A = ui_torque_current_a;
MotorFOC_SetTorqueCurrentA(iq_A);

// Display
float iq_actual = MotorFOC_GetIq();

## 9.4 Position command

// Absolute mechanical revolutions
MotorFOC_SetPositionRevolutions(1.25f);

// Or raw encoder counts
MotorFOC_SetPositionCounts(5000);

## 9.5 Stepper absolute or relative move

// Use a service-owned motion handle, not a local UI object
TMC2130Motion_MoveTo(&g_stepper_motion, 3200.0f);
TMC2130Motion_MoveBy(&g_stepper_motion, -400.0f);

// Display
float pos = TMC2130Motion_GetPositionSteps(&g_stepper_motion);
float vel = TMC2130Motion_GetVelocityStepsS(&g_stepper_motion);

## 9.6 Stepper follows BLDC position

// Example: 200 full steps/rev, 16 microsteps, gear ratio 1:1
float stepper_steps_per_bldc_rev = 200.0f * 16.0f;
float scale = stepper_steps_per_bldc_rev / (float)MOTOR_FOC_ENCODER_CPR;

TMC2130Motion_EnableBldcSync(&g_stepper_motion, scale, 0.0f);

// Periodic owner task continues calling:
TMC2130Motion_Task(&g_stepper_motion, 0.001f);

## 9.7 Stop and emergency stop

// Normal controlled stop
MotorFOC_RequestDisarm();
TMC2130Motion_Stop(&g_stepper_motion);

// Emergency stop
MotorFOC_RequestEmergencyStop();
TMC2130Motion_EmergencyStop(&g_stepper_motion);

# 10. Blocking and call-context warnings

| API pattern | Risk | Recommendation |
| --- | --- | --- |
| Buzzer_Beep() | Uses HAL_Delay(); blocks caller. | Avoid in TouchGFX callbacks. Use Buzzer_ToneHz + timer/service. |
| TMC2130_MoveBlocking() | Blocks until duration expires and estimates step count by time. | Use TMC2130Motion_* for UI motion. |
| Raw TMC2130_WriteReg() | Can change driver configuration while motor is active. | Restrict to engineering/debug screens. |
| MotorFOC_Init/Start/MotorFOC() | Lifecycle/owner functions. | Do not call from UI callbacks. |
| HAL GPIO/timer direct writes | Bypasses ownership model. | Use MotorFOC/TMC2130Motion/service APIs. |

# 11. Suggested UI service facade

For TouchGFX, it is cleaner to expose a small application service facade instead of calling low-level modules directly from every screen.

// Suggested app_ui_api.h facade, not currently required:
void AppUI_SetMotorMode(MotorFOC_Mode_t mode);
void AppUI_SetMotorSpeedRPM(float rpm);
void AppUI_SetMotorTorqueA(float iq_A);
void AppUI_SetMotorPositionRev(float rev);

void AppUI_StepperMoveTo(float steps);
void AppUI_StepperMoveBy(float steps);
void AppUI_StepperStop(void);
void AppUI_EnableStepperBldcSync(float scale, float offset);
void AppUI_DisableStepperBldcSync(void);

MotorFOC_State_t AppUI_GetMotorState(void);
float AppUI_GetMotorSpeedRPM(void);
float AppUI_GetStepperPositionSteps(void);
TMC2130Motion_State_t AppUI_GetStepperState(void);

This facade can hide global handles, protect call context, and keep TouchGFX screens independent from firmware internals.

# 12. Compact API index

| Category | APIs |
| --- | --- |
| BLDC commands | MotorFOC_SetMode, MotorFOC_SetCommand, MotorFOC_SetSpeedRPM, MotorFOC_SetTorqueCurrentA, MotorFOC_SetPositionCounts, MotorFOC_SetPositionRevolutions |
| BLDC requests | MotorFOC_RequestArm, RequestDisarm, RequestEmergencyStop, RequestZeroEncoder, RequestStop, Arm, Disarm, Stop |
| BLDC getters | MotorFOC_GetState, GetMode, GetRequestedMode, GetIa/Ib/Ic, GetId/Iq, GetSpeedRPM, GetPositionCounts |
| TMC low-level | TMC2130_ConfigDefaults, Init, InitEx, WriteReg, ReadReg, Enable, SetDirection, SetStepFrequency, StartStepClock, StopStepClock, IsStepClockRunning, GetStepFrequencyHz, MoveBlocking, ReadGSTAT, ClearGSTAT, ReadDRVStatus, ReadDiag0/1, ReadDcIn |
| TMC motion | TMC2130Motion_DefaultLimits, Init, Enable, MoveTo, MoveBy, Stop, EmergencyStop, Task, EnableBldcSync, DisableBldcSync, GetState, GetPositionSteps, GetTargetSteps, GetVelocityStepsS |
| Support | Telemetry_Init, Telemetry_Update, Telemetry_SetFrameBufferAddress, Buzzer_Init, Buzzer_ToneHz, Buzzer_Stop, Buzzer_Beep, LED0/1/2 On/Off/Toggle, MX_TouchGFX_* |